Design and Implementation of a TinyML-Powered Framework for Real-Time Automotive Collision Prediction and CNG Cabin Leakage Detection

Quantum Core

Team leader name                                                                    Team members

(Dharineesh T.A.)                                         (Thamilarasan M.) (Aswath M.) (Dharshan R.)

### 1. Idea

- Project Title: EdgeCrash-AI: Real-Time Automotive Anomaly, CNG Leakage, and Collision Detection System Using Edge Impulse.

- Core Concept: A TinyML-powered smart vehicle safety framework designed to predict collisions, CNG/LPG gas leakages, and cabin fire hazards at the edge (microcontroller level).

- System Architecture: A Dual-MCU (ESP32) system that isolates low-level sensor data collection from high-level machine learning processing via wireless communication.

- Intelligent HMI: Runs an on-device Edge Impulse classifier to analyze real-time variables and output live safety statuses on an I2C 16x2 LCD display.

### 2. Current State of the Idea

- Hardware Integration: The featuring ESP32 DevKitC V4, HC-SR04 (Ultrasonic), SW-420 (Vibration), MQ-2 (Gas/CNG/Smoke), and MB102 power management modules.

- Testbed Prototyping: A calibrated toy-car platform is developed to simulate physical crash impacts, sudden bumper-to-obstacle distances, and CNG leak / cabin smoke density safely in a lab environment.

- Dataset Engineering: The Edge Impulse workspace is fully initialized, and specialized time-series data structures (normal.csv and anomaly.csv) have been structured for model training.

### 3. What Are the Innovations?

- Pattern-Based Machine Learning: Replaces primitive, error-prone threshold limits (fixed if-else conditions) with a trained TinyML Anomaly Detection model capable of recognizing complex accident and gas leakage footprints.

- Asynchronous Dual-Core Processing: Offloads heavy neural network inference to a dedicated AI ESP32 unit, ensuring that local sensor data capture on the vehicle remains uninterrupted during critical crash or CNG leak seconds.

- Multi-Sensor Edge Fusion: Seamlessly merges kinetic impact forces, spatial distance data, and inflammable CNG / atmospheric smoke levels on-chip to construct a multi-dimensional automotive safety shield.